Loop Service with Number Portability (LSNP) LSOG 6 Form Preparation Guide

V28.0

TABLE OF CONTENTS

SECTION										                  PAGE

HISTORY LOG	4

1.	GENERAL SECTION	4

2.	LSNP FORM DESCRIPTION	5

3.	LSNP FORM ENTRIES	5

3.2	ADMINISTRATIVE SECTION	6

1.	PON - Purchase Order Number	6

2.	VER - Version Identification	7

3.	AN - Account Number	8

4.	ATN - Account Telephone Number – Not currently used by CenturyLink	9

5.	LQTY - Loop Quantity	10

6.	NPQTY - Number Portability Quantity – Not currently used by CenturyLink	11

7.	PG_of_ - Page _ of _	12

3.3	SERVICE DETAILS SECTION	13

8.	LOCNUM - Location Number – Not currently used by CenturyLink	13

9.	LNUM - Line Number	14

10.	NPI - Number Portability Indicator – Not currently used by CenturyLink	15

11.	LNA - Line Activity	16

12.	LMT – Loop Modification Type – Not currently used by CenturyLink	17

13.	BTRL – Bridge Tap Removal Location – Not currently used by CenturyLink	18

14.	CKR - Customer Circuit Reference	19

15.	TSP - Telecommunications Service Priority	21

16.	LRN - Location Routing Number –Not currently used by CenturyLink	22

17.	TDT - Ten Digit Trigger	23

18.	SAN - Subscriber Authorization Number	24

19.	ECCKT - Exchange Company Circuit ID	25

20.      RESID – Response Identifier – Not currently used by CenturyLink	27

21.	CFA - Connecting Facility Assignment	28

22.	CCEA – Cross Connect Equipment Assignment – Not currently used by CenturyLink	30

23.	SYSTEM ID - System Identification– Not currently used by CenturyLink	31

24.	CABLE ID - Cable Identification– Not currently used by CenturyLink	32

25.	CBCID –Cross Box Cable Identification – Not currently used by CenturyLink	33

26.	SHELF - Shelf– Not currently used by CenturyLink	34

27.	SLOT - Slot– Not currently used by CenturyLink	35

28.	SPORT – Slot Port – Not currently used by CenturyLink	36

29.	RELAY RACK – Relay Rack– Not currently used by CenturyLink	37

30.	CHAN/PAIR - Channel/Pair– Not currently used by CenturyLink	38

31.	UNIT – Unit – Not currently used by CenturyLink	39

32.	JK CODE - Jack Code – Not currently used by CenturyLink	40

33.	JK NUM – Jack Number – Not currently used by CenturyLink	41

34.	JK POS - Jack Position – Not currently used by CenturyLink	42

35.	JR - Jack Request – Not currently used by CenturyLink	43

36.	NIDR – NID Request	44

36a.	POTSSPLIT – POTS Splitter – CenturyLink-Specific Field	45

36b.	POTSSPLITLOC – POTS Splitter Location – CenturyLink-Specific Field	46

36c.	CABCONNQTY – Cable Connection Quantity – CenturyLink Specific Field	47

36d.	CABCONNTYP – Cable Connection Type – CenturyLink Specific Field	48

36e.	CABCONN– Cable Connection - CenturyLink Specific Field	49

37.	IWJK - Inside Wire Jack Code	50

38.	IWJQ - Inside Wire Jack Quantity	51

39.	PORTED NBR – Ported Telephone Number	52

40.	TNP - Total Number of Paths	53

41.	CFTN - Call Forward To Number	54

42.	NPT - Number Portability Type	55

43.	RTI - Route Index	56

44.	NPTG - Number Portability Trunk Group	57

45.	LSCP – Local Service Provider Change Prohibited – Not currently used by CenturyLink	58

46.	BA – Blocking Activity	59

47.	BLOCK - Block	60

48.	FPI - Freeze PIC Indicator – Not currently used by CenturyLink	62

49.	LPIC - IntraLATA Pre-subscription Indicator Code	63

50.	TC OPT - Transfer of Call Options	64

51.	TC TO PRI - Transfer of Calls To Primary Number	65

51a.	TCID – Transfer of Calls To Identifier – CenturyLink Specific Field	66

51b.	TC Name – Transfer of Call To Name– CenturyLink Specific Field	69

52.	TC TO SEC - Transfer of Calls To Secondary Number	70

53.	TCID - Transfer of Calls To Identifier	71

54.	TC NAME - Transfer of Calls To Name	74

55.	TC PER - Transfer of Calls Period	75

56.	LEAN - Line Existing Account Number – Not currently used by CenturyLink	76

57.	LEATN - Line Existing Account Telephone Number– Not currently used by CenturyLink	77

58.	REMARKS - Remarks	78

58a.	Manual IND – Manual Indicator – CenturyLink-Specific Field	79

59.	TCMI – Transfer of Call Message Indicator – Not currently used by CenturyLink	80

60.	TCMESS – Transfer of Calls Message – Not currently used by CenturyLink	81

3.4	ALPHANUMERIC GLOSSARY	82

Loop Service with Number Portability Form – Numbered

## HISTORY LOG

The History Log for this document may be accessed from the Table of Contents at the following web site: http://www.centurylink.com/wholesale/clecs/lsog.html. The link to the downloadable History Log has been indented under Loop Service with Number Portability (LSNP).

## 1.	GENERAL SECTION

1.1	The LSNP Form must always be associated with the Local Service Request (LSR) Form and the End User (EU) Form that is necessary for the provisioning of this request.

1.2	The LSOG Local Service Ordering Overview contains complete criteria for placing orders in the “Ordering Criteria Matrix and a “Product Groups Table” identifying the products that may be requested.  This document is accessed from the following Web site: http://centurylink.com/wholesale/clecs/lsog.html

1.3	Usage Strips or Usage Matrices are used for each field that is needed to complete the LSNP form.  The following Usage Matrix displays all of the products that are specific to this form.  The LSR Activity Types (ACT) that are allowed by product display an asterisk (*).  When an Activity Type is blank, a field entry is prohibited.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | * |  |
| Unbundled Local Loop/Local Number Portability (LNP) | * | * |
| Unbundled Loop Split/Local Number Portability (LNP) | * | * |

## 2.	LSNP FORM DESCRIPTION

2.1 	All information required for ordering Loop Service with Number Portability is provided for in the various fields contained within the LSNP Form.

## 3.	LSNP FORM ENTRIES

3.1	Included at the end of this document is an example of the LSNP Form with numbered fields to correspond with the Field Numbers within this document.  Fields/entries that are “Not currently used by CenturyLink” are shaded gray to indicate they are not to be used. This numbered form is intended as a guide when using this document.

CenturyLink-Specific Forms should be used when processing orders with CenturyLink™, as these forms contain:

- OBF fields which are used by CenturyLink

- OBF fields which are “Not currently used by CenturyLink” and shaded gray to indicate they are not to be used

- CenturyLink-Specific fields necessary to process your request

Paragraph 3.4, Alphanumeric Glossary, contains an alphabetic listing of the LSNP Form fields with a cross-reference to the field numbers.

3.1.1	 The LSNP is prepared by the CLEC/Reseller and is submitted to the CenturyLink SC (Service Center) for the ordering of local service. The term "SC" referenced throughout the LSNP practice is used so as to represent the organization that processes a customer's request for local service.

3.2	Downloadable LSNP Form

A downloadable blank LSNP Form is available on the CenturyLink Wholesale Web page at: http://www.centurylink.com/wholesale/forms/lsnp.html

The blank form can be populated in Microsoft Excel with a minimum system requirement of Office 97. The blank form must be downloaded and saved before populating.  Additionally, the blank form may be printed and submitted with handwritten field entries.

Each field on the LSNP Form is displayed in the same sequence identified in the accompanying LSOG 6 Form Preparation Guide.  The Field Name is identified above the field and below it are “hash” marks that indicate the number of Data Characteristics allowed in the individual field.

To begin populating, place your cursor in the field and begin typing your entry.  If more than the allowed numbers of characters are entered in a field, an error message will pop-up advising you that “The value you entered is not valid.”  Click “Retry”, check LSOG for the allowable character length, and re-enter.  You can move to another field by using the “Tab” or physically moving your cursor to your next entry (fields that are shaded gray are not accessible).

When the forms for your entire order are completed, fax them to CenturyLink for processing.

## 3.2	ADMINISTRATIVE SECTION

| ### PON - Purchase Order Number |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the CLEC’s unique purchase-order or requisition number that authorizes the issuance of this request. |  |

USAGE:

IMA XML/GUI: This field is not used by CenturyLink.

NOTE: The PON entry is derived from the LSR form.

Manual/Fax: This field is required.

The data in this field must match the PON field on the LSR form.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | R | R |
| Unbundled Loop Split/Local Number Portability (LNP) | R | R |

| DATA CHARACTERISTICS: | 16 alphanumeric characters |
| --- | --- |

| EXAMPLE: | 8 | 2 | 4 | Z | 9 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### VER - Version Identification |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the CLEC’s version number. |  |

| NOTE 1: | Any reissuance can use this entry to uniquely identify the form from any other version. |
| --- | --- |

| VALID ENTRIES: |  |
| --- | --- |
|  | AA - ZZ 00 - 99 Blank |

USAGE:

IMA XML/GUI: This field is not used by CenturyLink.

NOTE: The VER entry is derived from the LSR form.

Manual/Fax: This field is optional.

The data in this field must match the VER field on the LSR form.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | O |  |
| Unbundled Local Loop/Local Number Portability (LNP) | O | O |
| Unbundled Loop Split/Local Number Portability (LNP) | O | O |

| DATA CHARACTERISTICS: | 2 alphanumeric characters |
| --- | --- |

| EXAMPLE: | A | A |
| --- | --- | --- |

| ### AN - Account Number |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the main account number assigned by CenturyLink. |  |

VALID ENTRIES:

- Account Number

N = New Account Number

USAGE:

IMA XML/GUI: This field is not used by CenturyLink.

NOTE: The AN entry is derived from the LSR form.

Manual/Fax: This field is required.

The data in this field must match the AN field on the LSR form.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | R | R |
| Unbundled Loop Split/Local Number Portability (LNP) | R | R |

| DATA CHARACTERISTICS: | 16 alphanumeric characters |
| --- | --- |

| EXAMPLES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 0 | 3 | - | 5 | 5 | 5 | - | 1 | 2 | 1 | 2 |  |  |  |  |

| ### ATN - Account Telephone Number – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the account telephone number assigned by the NSP. |  |

| ### LQTY - Loop Quantity |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the quantity of loops involved in this service request. |  |

USAGE:	This field is required.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | R | R |
| Unbundled Loop Split/Local Number Portability (LNP) | R | R |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - This entry must equal the number of Loops to be created plus the total number of additional loops, which may differ from the total number of LNUMs. For example, an existing End-user record may indicate ten TNs with associated features and facilities. - Examples: 1. A request asks for ten loops and all ten TNs to port. In this case, the LQTY = 10 and the LNUMs = 10. 2. A request can also port ten TNs with ten Loops and add an additional five Loops. In this case, the LQTY = 15 and the LNUMs = 10. |

| DATA CHARACTERISTICS: | 4 numeric characters |
| --- | --- |

| EXAMPLE: | 0 | 8 | 0 | 0 |
| --- | --- | --- | --- | --- |

| ### NPQTY - Number Portability Quantity – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the quantity of ported numbers involved in this service request. |  |

| ### PG_of_ - Page _ of _ |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the page number and total number of pages contained in this request. |  |

USAGE:

IMA XML/GUI: This field is not used by CenturyLink.

NOTE: The PG_ of_ entry is derived from the LSR form.

Manual/Fax: This field is required.

The data in this field must match the PG_of_ field on the LSR form.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | R | R |
| Unbundled Loop Split/Local Number Portability (LNP) | R | R |

| DATA CHARACTERISTICS: | 4 numeric characters |
| --- | --- |

| EXAMPLE: | PG |  | 1 | of |  | 2 |
| --- | --- | --- | --- | --- | --- | --- |

## 3.3	SERVICE DETAILS SECTION

This is a repeating section.  LNUM through POTSSPLITLOC may repeat as a group.

| ### LOCNUM - Location Number – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the service location number for the service requested. |  |

| ### LNUM - Line Number |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the line or trunk as a unique number and each additional occurrence as a unique number. |  |

| NOTE 1: | The LNUM is customer assigned and is returned on the Local Response to the ordering customer. |
| --- | --- |
| NOTE 2: | Once LNUM is generated it cannot be changed and is retained through completion of the request. |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - LNUM 0001 should always be the primary Account Number. LNUM 0001 will be processed as the primary AN and will be the number published if it is associated with a directory listing. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |

USAGE: This field is required.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | R | R |
| Unbundled Loop Split/Local Number Portability (LNP) | R | R |

| DATA CHARACTERISTICS: | 4 numeric characters |
| --- | --- |

| EXAMPLE: | 0 | 0 | 0 | 1 |
| --- | --- | --- | --- | --- |

| ### NPI - Number Portability Indicator – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the status of the telephone number being ported. |  |

| ### LNA - Line Activity |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the activity involved at the line level. |  |

VALID ENTRIES:

| N | = | New loop |
| --- | --- | --- |
| C | = | Change an existing account, e.g., rearrangement, partial disconnect or addition – Not currently used by CenturyLink |
| D | = | Disconnect |
| M | = | Inside move of the physical termination within a building – Not currently used by CenturyLink |
| T | = | Outside move of end user location – Not currently used by CenturyLink |
| R | = | Record activity is for ordering administrative changes – Not currently used by CenturyLink |
| V | = | Conversion as Specified |

USAGE: This field is required.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | R | R |
| Unbundled Loop Split/Local Number Portability (LNP) | R | R |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop | - If ACT = Z, then LNA = V, D or N. - If ACT = V or Z, then LNA = V or D if CHC = B. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - If ACT = V, then LNA = V, D or N. - If LNA = D, then TN cannot be the BTN unless the NAN is populated or involve directory listing changes. - If LNA is populated, TN must be populated. |
| 3 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |
| 4 | - Unbundled Loop Split/Local Number Portability (LNP) | - If ACT = Z or V, then LNA = V, D or N. - If ACT = V or Z, then LNA = V or D if CHC = B. |

| DATA CHARACTERISTICS: | 1 alpha character |
| --- | --- |

| EXAMPLE: | N |
| --- | --- |

### 12. LMT – Loop Modification Type – Not currently used by CenturyLink

Identifies the loop changes involved in this service request.

### 13. BTRL – Bridge Tap Removal Location – Not currently used by CenturyLink

Identifies the location of the bridged tap to be removed from the loop.

| ### 14. CKR - Customer Circuit Reference |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the circuit number assigned by the customer. |  |

| NOTE 1: | CKR is used by the customer as a cross reference to the provider circuit ID and in many cases to identify the customer’s end-to end service. |
| --- | --- |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - An entry is required when NPT = A or C. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Loop Split/Local Number Portability (LNP) | - This entry is required when Directory Listings are included with a LSR and must be formatted in a NPA-NXX-LLLL TN format. - If CHC = B, this entry is required, and must be formatted in a NPA-NXX-LLLL TN format. - For Information Only: If CHC=B, this entry should equal the working TN at the end-users location. |
| 3 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |

| DATA CHARACTERISTICS: | 36 alphanumeric characters |
| --- | --- |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - Virgule (/) is not a valid character. |

| EXAMPLES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L | 0 | 0 | 0 | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 2 | 9 | 3 | 3 | 2 | 1 | 8 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 15. TSP - Telecommunications Service Priority |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the provisioning and restoration priority as defined under the TSP Service Vendor Handbook. |  |

| NOTE 1: | These codes are assigned by the TSP Program Office. |
| --- | --- |

| VALID ENTRIES: A dash is required in position 10 |  |
| --- | --- |
|  | Nine Character TSP Control Identifier |
|  | One Character Provisioning Priority Level (E, 0-5) |
|  | One Digit Restoration Priority Level (0-5) |

| NOTE 1: | A TSP code ending in “00” indicates “revocation”, the removal of a previously assigned TSP code. |
| --- | --- |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | O |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Loop Split/Local Number Portability (LNP) | - When populated, this field will be validated against valid values. - LNUM through POTSSPLITLOC may repeat as a group. |

| DATA CHARACTERISTICS: | 12 alphanumeric characters |
| --- | --- |

| EXAMPLE: | T | S | P | 1 | 2 | 3 | 4 | 5 | C | - | E | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 16. LRN - Location Routing Number –Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies a number used to uniquely identify a switch that has ported numbers and is used to route a call to the switch that owns the NPA-NXX portion of the LRN. |  |

| ### 17. TDT - Ten Digit Trigger |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the request for the activation of a ten-digit trigger for local routing number portability. |  |

VALID ENTRIES:

Y = Yes

| USAGE: | This field is conditional. |
| --- | --- |

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - An entry of Y is required when NPT = D. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |

| DATA CHARACTERISTICS: | 1 alpha character |
| --- | --- |

| EXAMPLE: | Y |
| --- | --- |

| ### 18. SAN - Subscriber Authorization Number |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies a number equivalent to the end user Purchase Order Number. |  |

| NOTE 1: | This may, at the option of the customer, be a requirement when providing service to some governmental agencies. |
| --- | --- |

USAGE:	This field is optional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | O |  |
| Unbundled Local Loop/Local Number Portability (LNP) | O | O |
| Unbundled Loop Split/Local Number Portability (LNP) | O | O |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - This entry represents the Subscriber Authorization Number. - For Information Only - An entry is required if 1st character of TOS = 3. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |

| DATA CHARACTERISTICS: | 30 alphanumeric characters |
| --- | --- |

| EXAMPLE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | B | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ### 19. ECCKT - Exchange Company Circuit ID |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies a provider’s circuit identification. |  |

| NOTE 1: | The format of the field is defined by the provider. |
| --- | --- |
| NOTE 2: | The layout of the field may be defined by the COMMON LANGUAGE standards. |

VALID ENTRIES:

Circuit ID Number Format

Facility ID Format – Not currently used by CenturyLink

Telephone Number Format – Not currently used by CenturyLink

Serial Number Format – Not currently used by CenturyLink

| RULE # | Products | Business Rules: |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | Examples: 29.LXFU123456..MS (Central) 3.LXFU123456..PN (Western) 5.LXFU123456..NW(Eastern) |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - This entry represents the CenturyLink Circuit ID which CenturyLink will assign and FOC to the CLEC. - This entry is required on all requests after CenturyLink makes the initial assignment. All components within the ID should be delimited by periods. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |

| DATA CHARACTERISTICS: | 42 alphanumeric characters |
| --- | --- |

| 2 | 9 | . | L | X | F | U | 1 | 2 | 3 | 4 | 5 | 6 | . | . | M | S |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 3 | . | L | X | F | U | 1 | 2 | 3 | 4 | 5 | 6 | . | . | P | N |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 5 | . | L | X | F | U | 1 | 2 | 3 | 4 | 5 | 6 | . | . | N | W |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 20. RESID – Response Identifier – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  | Identifies the response number assigned by the provider to relate associated transactions. |  |  |

| ### 21. CFA - Connecting Facility Assignment |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the provider carrier system and channel to be used. |  |

| NOTE 1: | The format and structure of this field is defined by ANSI in document T1.238, Identification of Telecommunication Facilities for the North American Telecommunications System. The Facility identifier consists of the following elements: |
| --- | --- |

| 1. | Facility Designation – A code that, for a specific type of facility, uniquely identifies a path between two network nodes (variable length, 1-5 alphanumeric characters). |
| --- | --- |
| 2. | Facility Type – A code that describes a type of facility when it is other than a single baseband channel on cable. Valid entries are outlined in Telcordia Technologies practice BR 795-450-100 (variable length, 1-6 alphanumeric characters). |
| 3. | Channel/Pair/Time Slot – A code that identifies a specific assignable portion of a facility (variable length, 1-5 alphanumeric characters). |
| 4. | Location A – A standardized code that uniquely identifies the location of facility terminal A, which has the lower in alpha/numeric sequence of the two facility location codes. Valid values are outlined in Telcordia Technologies practice BR-795-100-100 (8 or 11 alphanumeric characters). |
| 5. | Location Z – A standardized code that uniquely identifies the location of facility terminal Z, which has the higher in alpha/numeric sequence of the two facility location codes. Valid values are outlined in Telcordia Technologies practice BR 795-100-100 (8 or 11 alphanumeric characters). |

| NOTE 2: | Either virgules (/) or periods (.) are used consistently as delimiters to separate all elements of the CFA. |
| --- | --- |

VALID ENTRIES:

| RULE # | Products | Business Rules |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - Valid CFA format = CABNM = 1 to 10 a/n CABTYP = 1 to 13 a/n (a dash is also valid) SLOT = 1 to 5 a/n LOCA = 1 to 11 a/n LOCZ = 1 to 11 a/n All fields should be separated by either a space ( ) or a virgule (/). Example: ALT01 VF-2WIRE 48 ALBQNMMN ALBQNMMNHJ1 Or ALT01/VF-2WIRE/48/ALBQNMMN/ALBQNMMNHJ1 |

21. CFA - Connecting Facility Assignment (continued)

| RULE # | Products | Business Rules |
| --- | --- | --- |
| 2 | - Unbundled Local Loop/Local Number Portability (LNP) | If the CABTYP value is not known, a placeholder value of 1 – 13 a/n characters is allowed. Placeholder not allowed for supplemental LSRs. For a supplemental where the original LSR contained a placeholder CFA, the correct CFA value appears in the FOC, and should be used to populate the field. |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | N |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Loop Split/Local Number Portability (LNP) | - This entry identifies the CenturyLink system and the channel to be used. An entry is required when the LNA = N or V, but is not applicable when the LNA = D. - When ACT = V or Z, then this field is required if CHC = B. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |

| DATA CHARACTERISTICS: | 54 alphanumeric characters |
| --- | --- |

| EXAMPLES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | / | T | 1 | / | 3 | / | B | S | T | N | M | A | G | T | C | G | 0 | / | B | S | T | N | M | A |
| M | T | K | 3 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A | L | T | 0 | 1 | / | V | F | - | 2 | W | I | R | E | / | 4 | 8 | / | A | L | B | Q | N | M | M | N | / |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | L | B | Q | N | M | M | N | H | J | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A | L | T | 0 | 1 |  | V | F | - | 2 | W | I | R | E | 4 | 8 |  | A | L | B | Q | N | M | M | N |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | L | B | Q | N | M | M | N | H | J | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| NOTE 1: | The second example shows the proper format for ranging channel assignments. |
| --- | --- |

| ### 22. CCEA – Cross Connect Equipment Assignment – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the physical point of termination at a collocation arrangement. |  |

| ### 23. SYSTEM ID - System Identification– Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the customer's system to be used in a collocation arrangement. |  |

| ### 24. CABLE ID - Cable Identification– Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the provider's cable to be connected to the customer's equipment in a central office location. |  |

| ### 25. CBCID –Cross Box Cable Identification – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the provider’s cable to be connected to the customer’s terminal in a field location. |  |

| ### 26. SHELF - Shelf– Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the number assigned to the shelf within the relay rack. |  |

| ### 27. SLOT - Slot– Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the specific connection slot of the shelf to be used. |  |

| ### 28. SPORT – Slot Port – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the specific connection port of the slot to be used. |  |

| ### 29. RELAY RACK – Relay Rack– Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the bay/cabinet in a central office and may include the floor and aisle where the specific piece of equipment is located. |  |

| ### 30. CHAN/PAIR - Channel/Pair– Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the specific channel or pair within the provider's cable to be used for connection. |  |

| ### 31. UNIT – Unit – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the number assigned to a panel, shelf or case within the customer’s bay/cabinet indicated in the RELAY RACK field. |  |

| ### 32. JK CODE - Jack Code – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the standard code for the particular registered or non-registered jack used to terminate the service. |  |

| ### 33. JK NUM – Jack Number – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the number of the jack used on end user connections. |  |

| ### 34. JK POS - Jack Position – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the position in the jack that a particular service will occupy. |  |

| ### 35. JR - Jack Request – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates a request for a new jack. |  |

| ### 36. NIDR – NID Request |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates a request for a new Network Interface Device (NID). |  |

VALID ENTRIES:

Y = Yes

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | O |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - The NIDR is a Y if a NID is requested. An entry is only valid in states where the CLEC has negotiated inside wiring. NIDR is not applicable when LNA = D. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |

| DATA CHARACTERISTICS: | 1 alpha character |
| --- | --- |

| EXAMPLE: | Y |
| --- | --- |

### 36a. POTSSPLIT – POTS Splitter – CenturyLink-Specific Field

Indicates the cable connection point information for POTS Splitter Locations outside the CLEC’s cage.

USAGE:  This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | P |  |
| Unbundled Local Loop/Local Number Portability (LNP) | P | P |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - All instances of this field for one LSR must be in the same format, (starting with VDA). - LNUM through POTSSPLITLOC may repeat as a group. |
| 2 | - Unbundled Loop Split/Local Number Portability (LNP) | - Required if POTSSPLITLOC = O. - If CABCONN, CABCONNTYP, or CABCONNQTY is populated, this field is not allowed. |

| DATA CHARACTERISTICS: | 38 alphanumeric characters |
| --- | --- |

EXAMPLE:

| V | D | A | . | 1 | 2 | 3 | 4 | 5 | 6 | 7 | . | 1 | 2 | . | 1 | 2 | / | 1 | 2 | 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 36b. POTSSPLITLOC – POTS Splitter Location – CenturyLink-Specific Field

Indicates if the POTS Splitter is inside or outside the CLEC’s cage or at a remote location.

VALID ENTRIES:

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | I = Inside collocation cage O = Outside collocation cage |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | P |  |
| Unbundled Local Loop/Local Number Portability (LNP) | P | P |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Unbundled Loop Split/Local Number Portability (LNP) | - Required when splitting the Loop, at least one POTSSPLITLOC must be on the request in the Service Details Section. - Value of “R” in invalid. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - LNUM through POTSSPLITLOC may repeat as a group. |

| DATA CHARACTERISTICS: | 1 alpha character |
| --- | --- |

| EXAMPLE: | I |
| --- | --- |

### 36c. CABCONNQTY – Cable Connection Quantity – CenturyLink Specific Field

Indicates the number of cable connections being specified.

VALID ENTRIES:

0-9

USAGE:

IMA XML/GUI: This field is populated by CenturyLink.

Manual/Fax: This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | N | N |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - This field identifies the number of times CABCONNTYP and CABCONN repeat. |
| 2 | - Unbundled Loop Split/Local Number Portability (LNP) | - Required to be 1 when POTSSPOLITLOC = I and CABCONNTYP = B. - If POTSSPLIT is populated, then this field is not allowed. |

| DATA CHARACTERISTICS: | 1 numeric character |
| --- | --- |

| EXAMPLE: | 9 |
| --- | --- |

### 36d. CABCONNTYP – Cable Connection Type – CenturyLink Specific Field

Indicates which Cable Connection Type is being specified in CABCONN.

VALID ENTRIES:

| RULE # | Products | Business Rules |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - First Character = D (DS0) - B = Voice |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | N | N |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Unbundled Loop Split/Local Number Portability (LNP) | - Required to be B when POTSSPLITLOC = I - If POTSSPLIT is populated, then this field is not allowed. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - This field and the next may repeat as a group up to 9 times. |

| DATA CHARACTERISTICS: | 2 alphanumeric character |
| --- | --- |

EXAMPLE:

| D |  |
| --- | --- |

### 36e. CABCONN– Cable Connection - CenturyLink Specific Field

Identifies the cable connection point information locations inside the CLECs cage.

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | R |  |
| Unbundled Local Loop/Local Number Portability (LNP) | N | N |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - All instances of this field for one LSR must be in the same format (starting with VDA). - This field and the next may repeat as a group up to 9 times. |
| 2 | - Unbundled Loop Split/Local Number Portability (LNP) | - Required when POTSSPLITLOC = I. - If POTSSPLIT is populated, then this field is not allowed. |

| DATA CHARACTERISTICS: | 38 alphanumeric character |
| --- | --- |

EXAMPLE:

| V | D | A | . | 0 | 0 | 0 | 0 | 0 | 0 | 5 | . | 0 | 1 | . | 0 | 3 | - | 0 | 0 | 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EXAMPLE for: Unbundled Distribution Loop (UDL)/Local Number Portability (LNP)

| F | C | P | 4 | 5 | E | T | , | 6 | 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The following 2 fields repeat, as a group, a maximum of 4 times within the repeating Service Details Section.

| ### 37. IWJK - Inside Wire Jack Code |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the standard code for the type of jack requested for inside wiring. |  |

| NOTE 1: | Familiarization with the FCC's registration rules is requisite for all parties involved for the determination of the proper jack code for a given registered service. Registered jacks used to terminate category 1 and 3 services begin with the designation "RJ". |
| --- | --- |
| NOTE 2: | When multiple lines are terminating in one multi-line jack, the IWJK and IWJQ fields should only be populated for the first line. Jacks may be ordered on a line by line basis. |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - Valid only in states where CLEC has negotiated inside wiring. Not applicable when LNA = D. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - The following 2 fields repeat, as a group, a maximum of 4 times within the repeating Service Details Section. |

USAGE:	This field is optional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | O |  |
| Unbundled Local Loop/Local Number Portability (LNP) | O | O |
| Unbundled Loop Split/Local Number Portability (LNP) | O | O |

| NOTE 1: | Required when the IWJQ field is populated, otherwise prohibited. |
| --- | --- |

| DATA CHARACTERISTICS: | 5 alphanumeric characters |
| --- | --- |

| EXAMPLE: | R | J | 2 | 1 | X |
| --- | --- | --- | --- | --- | --- |

| ### 38. IWJQ - Inside Wire Jack Quantity |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the number of jacks requested for inside wiring. |  |

| NOTE 1: | When multiple lines are terminating in one multi-line jack, the IWJK and IWJQ fields should only be populated for the first line. Jacks may be ordered on a line by line basis. |
| --- | --- |

VALID ENTRIES:

01 – 99

| USAGE: | This field is conditional. |
| --- | --- |

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - If the field IWJK is populated, then IWJQ must be greater than zero. If the field IWJK is not populated then IWJQ must not be populated. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - The following field repeats, as a group with IWJK, a maximum of 4 times within the repeating Service Details Section. |

| DATA CHARACTERISTICS: | 2 numeric characters |
| --- | --- |

| EXAMPLE: | 0 | 1 |
| --- | --- | --- |

| ### 39. PORTED NBR – Ported Telephone Number |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the telephone number (TN) and/or consecutive range of TNs residing in the same switch, that are to be ported. |  |

| NOTE 1: | A range of numbers can be presented as a block of numbers or multiple iterations. |
| --- | --- |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Loop Split/Local Number Portability (LNP) | - PORTED NBR is NOT required if LNA = N. |

| DATA CHARACTERISTICS: | 12 alphanumeric characters (dashes are required in positions 4 and 8) |
| --- | --- |

| EXAMPLES: |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 0 | 3 | - |  | 6 | 9 | 9 | - | 7 | 0 | 0 | 0 |

| ### 40. TNP - Total Number of Paths |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the total number of talk paths, including the initial path, associated with the ported number. |  |

| VALID ENTRIES: |  |
| --- | --- |
|  | 001-999 |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | An entry is required when NPT = B. |

| DATA CHARACTERISTICS: | 3 numeric characters |
| --- | --- |

| EXAMPLE: | 0 | 0 | 1 |
| --- | --- | --- | --- |

| ### 41. CFTN - Call Forward To Number |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the telephone number to which calls will be directed. |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| NOTE 1: | An entry is required when the NPT field is “B.” |
| --- | --- |

| DATA CHARACTERISTICS: | 12 alphanumeric characters (dashes are required in positions 4 and 8) |
| --- | --- |

| EXAMPLES: | 3 | 0 | 3 | - | 5 | 5 | 5 | - | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 8 | 0 | 0 | - | 5 | 5 | 5 | - | 0 | 0 | 0 | 0 |

| ### 42. NPT - Number Portability Type |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the type of number portability for this request. |  |

VALID ENTRIES:

| A | = | DID |
| --- | --- | --- |
| B | = | RCF |
| C | = | Route Index |
| D | = | Local Routing Number (LRN) |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - Distinguishes between INP and LNP. For INP, either A, B, or C is used. For LNP, D is used. Required if LNA = V. When LNA = D, NPT is not populated. - This field only needs to be populated when the PORTED NBR is populated. - If NPT is not populated, disconnect the TN during Port Out. |

| DATA CHARACTERISTICS: | 1 alpha character |
| --- | --- |

| EXAMPLE: | A |
| --- | --- |

| ### 43. RTI - Route Index |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the routing index to be used by the provider's switching equipment to forward/port the provider's telephone number to the customer's non-RCF trunk group. |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | An entry is required when NPT = A or C. An entry is not applicable when the request is for RCF (NPT = B) or LRN (NPT = D). |

| DATA CHARACTERISTICS: | 6 alphanumeric characters |
| --- | --- |

| EXAMPLE: | 2 | 1 | 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |

| ### 44. NPTG - Number Portability Trunk Group |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the two six code (TSC) of a dedicated trunk group, from the porting switch to the customer's point of interface (POI), used to complete NP calls. |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | An entry is required when NPT = A or C. An entry is not applicable when the request is for RCF (NPT = B) or LRN (NPT = D). |

| DATA CHARACTERISTICS: | 8 alphanumeric characters |
| --- | --- |

| EXAMPLE: | A | K | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 45. LSCP – Local Service Provider Change Prohibited – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies that the end user has requested the option of prohibiting the change of their current service provider or removing the option. |  |

| ### 46. BA – Blocking Activity |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the activity for the blocking of calls. |  |

| VALID ENTRIES: |  |  |  |
| --- | --- | --- | --- |
|  | A | = | Add – Not currently used by CenturyLink |
|  | D | = | Delete |
|  | E | = | End state of the account |
|  | N | = | No Change – Not currently used by CenturyLink |
|  | Z | = | Remove all blocking |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | O |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - An entry is optional when NPT = A, B, C. If NPT=D then BA values are prohibited. BA = E indicates the end state of the account. Existing block values not specified will be removed. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Loop Split/Local Number Portability (LNP) | - If ACT = V or Z and blocking is part of an existing line, BA = E is the only valid value allowed to indicate the end state of the account. - If ACT = V or Z and BA is not populated, then blocking will be removed from the line. |
| 3 | - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - If ACT = V and blocking is part of an existing line, BA = E is the only valid value allowed to indicate the end state of the account. - If ACT = V, BA is not populated, then blocking will be removed from the line. |

| DATA CHARACTERISTICS: | 1 alpha character |
| --- | --- |

| EXAMPLE: | D |
| --- | --- |

| ### 47. BLOCK - Block |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the type of blocking on the telephone number. |  |

| VALID ENTRIES: |  |  |  |
| --- | --- | --- | --- |
|  | This field can contain multiple blocking values based on the Negotiated Business Rules. When requesting blocking, commas, spaces and special characters are not allowed. |  |  |
|  | A | = | No collect and third party |
|  | B | = | No third party |
|  | C | = | No collect call |
|  | D | = | No 1+ |
|  | E | = | No 0+ Local |
|  | F | = | No 1+ and 0+ Local |
|  | G | = | No 011 (International) |
|  | H | = | No Directory Assistance Call Completion (DACC) |
|  | J | = | No 700 – Not currently used by CenturyLink |
|  | K | = | No 976 |
|  | L | = | No 915– Not currently used by CenturyLink |
|  | M | = | No 900 |
|  | N | = | No casual calling |
|  | P | = | No N11 (except 911 and 411) – Not currently used by CenturyLink |
|  | Q | = | No 411 |
|  | R | = | No incoming (CENTREX) – Not currently used by CenturyLink |
|  | S | = | No 976, 900-303, 900-505– Not currently used by CenturyLink |
|  | T | = | No 900-303– Not currently used by CenturyLink |
|  | W | = | No 7 digit toll (CENTREX) – Not currently used by CenturyLink |
|  | Y | = | No 500– Not currently used by CenturyLink |
|  | Z | = | All– Not currently used by CenturyLink |
|  | This field can contain multiple blocking values based on the Negotiated Business Rules. |  |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

47.	BLOCK – Block (continued)

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | - If NPT not = D and BA = E, A or D, then BLOCK is required. - If NPT = D then BLOCK values are prohibited. - If BA = Z then BLOCK values are prohibited. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | The following BLOCK values block billing to the end-user’s account: A = No collect and third party (Billed Number Screening) B = No third party (Billed Number Screening) C = No collect call (Billed Number Screening) If BLOCK contains A, BLOCK cannot contain B or C. If BLOCK contains B, BLOCK cannot contain A or C. If BLOCK contains C, BLOCK cannot contain A or B |

| DATA CHARACTERISTICS: | 32 alpha characters |
| --- | --- |

| EXAMPLES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | F | H | J |  | K |  |  |  |  |  |  |  |  |  |  |  |

| Q |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 48. FPI - Freeze PIC Indicator – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the customer's requested freeze option for the LPIC. |  |

| ### 49. LPIC - IntraLATA Pre-subscription Indicator Code |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the Pre-subscription Indicator Code (PIC) of the carrier the customer has selected for IntraLATA traffic for the ported telephone number. |  |

| VALID ENTRIES: |  |  |  |
| --- | --- | --- | --- |
|  | Valid LPIC code |  |  |
|  | NONE = No Carrier Selected |  |  |
|  | NA | = | Not applicable – Not currently used by CenturyLink |
|  | DFLT | = | Default (CenturyLink Carrier Selected) |
|  | NC | = | No change– Not currently used by CenturyLink |
|  | UNDC | = | Undecided– Not currently used by CenturyLink |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | O |  |
| Unbundled Local Loop/Local Number Portability (LNP) | O | O |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | An entry is not applicable when NPT = D or when LNA = D. |

| DATA CHARACTERISTICS: | 4 alphanumeric characters |
| --- | --- |

| EXAMPLE: | 0 | 4 | 4 | 0 |
| --- | --- | --- | --- | --- |

| ### 50. TC OPT - Transfer of Call Options |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the type of transfer of call option the end user has requested. |  |

VALID ENTRIES:

| Custom Code Identifier |  |  |
| --- | --- | --- |
| N | = | None (basic intercept message - no new number referral) |
| S | = | Standard (i.e. forwarded to the new number) |
| T | = | Split Transfer of Calls (i.e. forwarded to multiple new numbers) |
| O | = | Operator Referral – Not currently used by CenturyLink |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | If LNA = D, then an entry is required. |

| DATA CHARACTERISTICS: | 1 alphanumeric characters |
| --- | --- |

| EXAMPLES: | S |
| --- | --- |

|  | T |
| --- | --- |

| ### 51. TC TO PRI - Transfer of Calls To Primary Number |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the telephone number to which calls are to be referred. |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | If TC OPT is populated with S or T, then an entry is required. |

| DATA CHARACTERISTICS: | 12 alphanumeric characters (dashes are required in positions 4 and 8) |
| --- | --- |

| EXAMPLE: | 2 | 0 | 1 | - | 5 | 5 | 5 | - | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 51a. TCID – Transfer of Calls To Identifier – CenturyLink Specific Field

Identifies the intercept message requested for a primary telephone number associated with intercept services.

VALID ENTRIES:

| TCID ENTRY | TYPE OF SERVICE | DESCRIPTION | AUTOMATED MESSAGE |
| --- | --- | --- | --- |
| Blank (GUI or manual) | Basic | Disconnect message, No referral | “This number, (#), has been disconnected.” (REPEAT) |
| 04 | Basic | Number Change, Non-List, No referral | “The number, (#), has been changed to a non-listed number.” (REPEAT) |
| 05 | Basic | Number Change, Non-Published, No referral | “The number, (#), has been changed to a non-published number.” (REPEAT) |
| 06 | Basic | Number Change, Not yet connected, No referral | “The number, (#), is not yet connected. (REPEAT) Please try your call again.” |
| 25 | New Number Referral | Disconnect, Referral to New Number | “The number, (#), has been disconnected. Calls are being taken by (#).” (REPEAT) |
| 30 | Basic | Temporary Disconnect, No referral customer request | “At the customer’s request, (#), has been temporarily disconnected.” (REPEAT) |
| 39 | Basic | Temporary Disconnect, No referral - non-pay | “The number, (#), has been temporarily disconnected. (REPEAT) No further information is available about this number.” |
| 45 | New Number Referral | Temporary Disconnect, Referral to New Number | “At the customer’s request, (#), has been temporarily disconnected. Calls are being taken by (#).” (REPEAT) |
| 55 | New Number Referral | Number Change, Referral to New Number | “The number, (#), has been changed to (#). (REPEAT) |
| 65 | New Number Referral | Temporary Number Change, Referral to Another Number | “The number, (#), has been temporarily changed to (#). (REPEAT) |

51a. TCID – Transfer of Calls To Identifier – CenturyLink Specific Field - continued

| TCID ENTRY | TYPE OF SERVICE | DESCRIPTION | AUTOMATED MESSAGE |
| --- | --- | --- | --- |
| 70 | Split Number Referral | Split Number Referral | - When an end user dials a number that has been disconnected, the end user will receive a live or automated message that provides them with the name and new number of each of the parties that were affiliated with the original number that the end user dialed. |

51a. TCID – Transfer of Calls To Identifier – CenturyLink Specific Field - continued

| TCID ENTRY | TYPE OF SERVICE | DESCRIPTION | AUTOMATED MESSAGE |
| --- | --- | --- | --- |
| 74 | Basic | TTY Basic Intercept, No referral – Not currently used by CenturyLink |  |
| 75 | New Number Referral | TTY Number Change, Referral to New Number – Not currently used by CenturyLink |  |
| 76 | New Number Referral | TTY Disconnect, Referral to New Number – Not currently used by CenturyLink |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | If TC OPT is populated with = T, then an entry is required. |

| DATA CHARACTERISTICS: | 2 numeric characters |
| --- | --- |

EXAMPLE:

| 2 | 5 |
| --- | --- |

### 51b. TC Name – Transfer of Call To Name– CenturyLink Specific Field

Identifies the name associated with TC TO PRI to which calls are to be referred when split transfer of calls is requested.

USAGE:  This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | If TC OPT is populated with = T, then an entry is required. |

| DATA CHARACTERISTICS: | 35 alphanumeric characters |
| --- | --- |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | Virgule (/) is not a valid character. |

EXAMPLE:

| A | R | T | H | U | R |  | R | O | B | I | N | S | O | N |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 52. TC TO SEC - Transfer of Calls To Secondary Number |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the secondary telephone number to which calls are to be referred on a split transfer of calls. |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | If TC OPT is populated with T, then an entry is required. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | Fields TC TO SEC, TCID, & TC NAME as a group represents a secondary set of information for Transfer of Calls Section; it repeats for as many “additional” parties requested. |

| DATA CHARACTERISTICS: | 12 alphanumeric characters (dashes are required in positions 4 & 8) |
| --- | --- |

| EXAMPLE: | 2 | 0 | 1 | - | 6 | 9 | 9 | - | 1 | 2 | 3 | 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 53. TCID - Transfer of Calls To Identifier |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the intercept message requested for a secondary telephone number associated with split transfer of calls. |  |

| NOTE 1: | The TCID should be sequential starting with the number one (1). The first TCID is associated to the TC TO PRI and the first occurrence of the TC NAME fields. |
| --- | --- |
| NOTE 2: | The second occurrence of TCID is associated to the first occurrence of the TC TO SEC and the second occurrence of the TC NAME fields. The TCID should incrementally increase by one (1) until all occurrences of TC TO SEC and TC NAME fields have been exhausted. |

VALID ENTRIES:

| TCID ENTRY | TYPE OF SERVICE | DESCRIPTION | AUTOMATED MESSAGE |
| --- | --- | --- | --- |
| Blank (GUI or manual) | Basic | Disconnect message, No referral | “This number, (#), has been disconnected.” (REPEAT) |
| 04 | Basic | Number Change, Non-List, No referral | “The number, (#), has been changed to a non-listed number.” (REPEAT) |
| 05 | Basic | Number Change, Non-Published, No referral | “The number, (#), has been changed to a non-published number.” (REPEAT) |
| 06 | Basic | Number Change, Not yet connected, No referral | “The number, (#), is not yet connected. (REPEAT) Please try your call again.” |
| 25 | New Number Referral | Disconnect, Referral to New Number | “The number, (#), has been disconnected. Calls are being taken by (#).” (REPEAT) |
| 30 | Basic | Temporary Disconnect, No referral customer request | “At the customer’s request, (#), has been temporarily disconnected.” (REPEAT) |
| 39 | Basic | Temporary Disconnect, No referral - non-pay | “The number, (#), has been temporarily disconnected. (REPEAT) No further information is available about this number.” |
| 45 | New Number Referral | Temporary Disconnect, Referral to New Number | “At the customer’s request, (#), has been temporarily disconnected. Calls are being taken by (#).” (REPEAT) |
| 55 | New Number Referral | Number Change, Referral to New Number | “The number, (#), has been changed to (#). (REPEAT) |
| 65 | New Number Referral | Temporary Number Change, Referral to Another Number | “The number, (#), has been temporarily changed to (#). (REPEAT) |

53. TCID - Transfer of Calls To Identifier – continued

| TCID ENTRY | TYPE OF SERVICE | DESCRIPTION | AUTOMATED MESSAGE |
| --- | --- | --- | --- |
| 70 | Split Number Referral | Split Number Referral | - When an end user dials a number that has been disconnected, the end user will receive a live or automated message that provides them with the name and new number of each of the parties that were affiliated with the original number that the end user dialed. |

53. TCID - Transfer of Calls To Identifier – continued

| TCID ENTRY | TYPE OF SERVICE | DESCRIPTION | AUTOMATED MESSAGE |
| --- | --- | --- | --- |
| 74 | Basic | TTY Basic Intercept, No referral – Not currently used by CenturyLink |  |
| 75 | New Number Referral | TTY Number Change, Referral to New Number – Not currently used by CenturyLink |  |
| 76 | New Number Referral | TTY Disconnect, Referral to New Number – Not currently used by CenturyLink |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | If TC OPT is populated with = T, then an entry is required. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | Fields TC TO SEC, TCID, & TC NAME as a group represents a secondary set of information for Transfer of Calls Section; it repeats for as many “additional” parties requested. |

| DATA CHARACTERISTICS: | 2 numeric characters |
| --- | --- |

| EXAMPLE: | 3 | 2 |
| --- | --- | --- |

| ### 54. TC NAME - Transfer of Calls To Name |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the name(s) associated with TC TO SEC fields to which calls are referred when split transfer of calls is requested. |  |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | If TC OPT is populated with = T, then an entry is required. |
| 2 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | Fields TC TO SEC, TCID, & TC NAME as a group represents a secondary set of information for Transfer of Calls Section; it repeats for as many “additional” parties requested. |

| DATA CHARACTERISTICS: | 35 alphanumeric characters |
| --- | --- |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | Virgule (/) is not a valid character. |

| EXAMPLES: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S | A | L | L | Y |  |  | J | O | N | E | S |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| A | L | L |  | O | T | H | E | R |  | N | A | M | E | S |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 55. TC PER - Transfer of Calls Period |  |  |  |
| --- | --- | --- | --- |
|  |  | Indicates the requested date that the transfer of calls, specified in the TC TO PRI field, is to be removed and the standard recorded announcement is to be provided. |  |

VALID ENTRIES:

| RULE # | Products | Valid Entries |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | - Metric Format - CCYYMMDD |

USAGE:	This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | Required if TC OPT is populated with = S or T. |

| DATA CHARACTERISTICS: | 8 alphanumeric characters |
| --- | --- |

| EXAMPLES: | 2 | 0 | 0 | 2 | 0 | 9 | 1 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

|  | 2 | 0 | 0 | 2 | 0 | 3 | 0 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| ### 56. LEAN - Line Existing Account Number – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the end user’s existing account number assigned by the current NSP and/or LSP. |  |

| ### 57. LEATN - Line Existing Account Telephone Number– Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies the end user’s existing account telephone number assigned by the old LSP. |  |

| ### 58. REMARKS - Remarks |  |  |  |
| --- | --- | --- | --- |
|  |  | Identifies a free flowing field that can be used to expand upon and clarify other data on this form. |  |

USAGE:	This field is optional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | O |  |
| Unbundled Local Loop/Local Number Portability (LNP) | O | O |
| Unbundled Loop Split/Local Number Portability (LNP) | O | O |

| DATA CHARACTERISTICS: | 255 alphanumeric characters |
| --- | --- |

EXAMPLE:

| D | I | S | C |  | S | E | R | V | I | C | E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 58a. Manual IND – Manual Indicator – CenturyLink-Specific Field

Indicates that the CLEC has provided information in the Remarks section that must be processed manually by the CenturyLink Service Center.

| RULE # | Products | Business Rule |
| --- | --- | --- |
| 1 | - Local Number Portability (LNP)/Unbundled Local Loop - Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) - Unbundled Loop Split/Local Number Portability (LNP) | MANUAL IND must be set to ‘Y’ by the CLEC if the REMARKS field contains information that must be processed manually by the CenturyLink Service Center or if any other rule indicates the MANUAL IND should be set to ‘Y’. |

VALID ENTIRES:

- Y – REMARKS must be processed manually

- N – REMARKS do not need manual attention

USAGE: This field is conditional.

| PRODUCT | ACTIVITY TYPE |  |
| --- | --- | --- |
|  | V | Z |
| Unbundled Distribution Loop (UDL)/Local Number Portability (LNP) | C |  |
| Unbundled Local Loop/Local Number Portability (LNP) | C | C |
| Unbundled Loop Split/Local Number Portability (LNP) | C | C |

| DATA CHARACTERISTICS: | 1 alphanumeric characters |
| --- | --- |

| EXAMPLES: | Y |
| --- | --- |

| ### 59. TCMI – Transfer of Call Message Indicator – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Associates the transfer of call message with the TCID of the telephone number for which the message is being requested. |  |

| ### 60. TCMESS – Transfer of Calls Message – Not currently used by CenturyLink |  |  |  |
| --- | --- | --- | --- |
|  |  | Allows for a personalized message when requesting the transfer of calls option. |  |

## 3.4	ALPHANUMERIC GLOSSARY

The following table is an alphanumeric cross-reference glossary of the Loop Service with Number Portability Form fields.

| LSNP FORM |  |  |
| --- | --- | --- |
| Field Abbreviation | Field # | Field Name |
| AN | 3 | Account Number |
| ATN | 4 | Acccount Telephone Number - Not currently used by CenturyLink |
| BA | 46 | Blocking Activity |
| BLOCK | 47 | Block |
| BTRL | 13 | Bridged Tap Removal Location – Not currently used by CenturyLink |
| CABCONN | 36e | Cable Connection – CenturyLink-Specific Field |
| CABCONNQTY | 36c | Cable Connection Quantity – CenturyLink-Specific Field |
| CABCONNTYP | 36d | Cable Connection Type – CenturyLink Specific Field |
| CABLE ID | 24 | Cable Identification - Not currently used by CenturyLink |
| CBCID | 25 | Cross Box Cable Identification – Not currently used by CenturyLink |
| CCEA | 22 | Cross Connect Equipment Assignment - Not currently used by CenturyLink |
| CFA | 21 | Connecting Facility Assignment |
| CFTN | 41 | Call Forward To Number |
| CHAN/PAIR | 30 | Channel/Pair - Not currently used by CenturyLink |
| CKR | 14 | Customer Circuit Reference |
| ECCKT | 19 | Exchange Company Circuit ID |
| FPI | 48 | Freeze PIC Indicator – Not currently used by CenturyLink |
| IWJK | 37 | Inside Wire Jack Code |
| IWJQ | 38 | Inside Wire Jack Quantity |
| JK CODE | 32 | Jack Code - Not currently used by CenturyLink |
| JK NUM | 33 | Jack Number - Not currently used by CenturyLink |
| JK POS | 34 | Jack Position - Not currently used by CenturyLink |
| JR | 35 | Jack Request – Not currently used by CenturyLink |
| LEAN | 56 | Line Existing Account Number - Not currently used by CenturyLink |
| LEATN | 57 | Line Existing Account Telephone Number - Not currently used by CenturyLink |
| LNA | 11 | Line Activity |
| LNUM | 9 | Line Number |
| LMT | 12 | Loop Modification Type – Not currently used by CenturyLink |
| LOCNUM | 8 | Location Number - Not currently used by CenturyLink |
| LPIC | 49 | IntraLATA Pre-subscription Indicator Code |
| LQTY | 5 | Loop Quantity |
| LRN | 16 | Location Routing Number - Not currently used by CenturyLink |
| LSCP | 45 | Local Service Provider Change Prohibited – Not currently used by CenturyLink |
| MANUAL IND | 58a | Manual Indicator – CenturyLink-Specific Field |
| NIDR | 36 | NID Request |
| NPI | 10 | Number Portability Indicator - Not currently used by CenturyLink |
| NPQTY | 6 | Number Portability Quantity - Not currently used by CenturyLink |
| NPT | 42 | Number Portability Type |

3.4	ALPHANUMERIC GLOSSARY (continued)

| LSNP FORM (continued) |  |  |
| --- | --- | --- |
| Field Abbreviation | Field # | Field Name |
| NPTG | 44 | Number Portability Trunk Group |
| PG_of_ | 7 | Page _ of _ |
| PON | 1 | Purchase Order Number |
| PORTED NBR | 39 | Ported Telephone Number |
| POTSSPLIT | 36a | POTS Splitter – CenturyLink-Specific Field |
| POTSSPLITLOC | 36b | POTS Splitter Location – CenturyLink-Specific Field |
| RELAY RACK | 29 | Relay Rack - Not currently used by CenturyLink |
| REMARKS | 58 | Remarks |
| RESID | 20 | Response Identifier – Not currently used by CenturyLink |
| RTI | 43 | Route Index |
| SAN | 18 | Subscriber Authorization Number |
| SHELF | 26 | Shelf - Not currently used by CenturyLink |
| SPORT | 28 | Slot Port – Not currently used by CenturyLink |
| SLOT | 27 | Slot - Not currently used by CenturyLink |
| SYSTEM ID | 23 | System Identification - Not currently used by CenturyLink |
| TCID | 51a | Transfer of Calls to Identifier – CenturyLink- Specific Field |
| TCID (Secondary Section) | 53 | Transfer of Calls to Identifier |
| TCMESS | 60 | Transfer of Calls Message - Not currently used by CenturyLink |
| TCMI | 59 | Transfer of Call Message Indicator - Not currently used by CenturyLink |
| TC NAME | 51b | Transfer of Calls to Name - CenturyLink-Specific Field |
| TC NAME (Secondary Section) | 54 | Transfer of Calls to Name |
| TC OPT | 50 | Transfer of Call Options |
| TC PER | 55 | Transfer of Calls Period |
| TC TO PRI | 51 | Transfer of Calls to Primary Number |
| TC TO SEC | 52 | Transfer of Calls to Secondary Number |
| TDT | 17 | Ten Digit Trigger |
| TNP | 40 | Total Number of Paths |
| TSP | 15 | Telecommunications Service Priority |
| UNIT | 31 | Unit - Not currently used by CenturyLink |
| VER | 2 | Version Identification |