| Ref | LSOG6 POP 120 REF | Field Name | N/R/C/O | Negotiated Business Rules | Field Lengths | Field Characteristics | Valid Values |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N = Not Req’d R = Required C = Conditional O = Optional |  |  |  |  |
|  |  | Raw Loop Data Query (RLDQ) |  |  |  |  |  |
|  |  | Administrative Section |  |  |  |  |  |
| RLDQ1 | 2 | TXNUM | R | Co-Provider generated and may be reused one month after initial inquiry. | 16 | a/n |  |
| RLDQ2 | 3 | D/TSENT XML: MSG_TIMESTAMP | R |  | 17 | a/n | CCYYMMDDHHMinMin Military Time |
| RLDQ3 | 4 | TXTYP | R |  | 1 | a/n | R = Raw Loop Data Query |
| RLDQ4 | 5 | TXACT | R |  | 1 | a/n | A = New Inquiry |
| RLDQ5 | 7 | CC | R |  | 4 | a/n |  |
|  |  | Raw Loop Data Query Section |  |  |  |  |  |
| RLDQ6 |  | TNADDRCKTIND | R | Telephone Number, Address, Circuit Indicator: This field indicated how the query is being done. | 1 | a/n | T = TN A = Address C = Circuit ID |
| RLDQ7 |  | ASGNIND | C | Assignment Indicator Required if TNADDRCKTIND = A. | 1 | a/n | A = Assigned U = Unassigned |
|  |  | The following two fields repeat as a group up to 24 times.. |  |  |  |  |  |
| RLDQ8 | 40 | QR* | C | Quantity Requested: This is the total of Telephone Numbers being queried on. Required if TNADDRCKTIND = T. Index of TN/rows in query. | 2 | n |  |
| RLDQ9 | 52 | WTN* | C | Required if TNADDRCKTIND = T. For Information Only: Ranges are not supported | 15 | a/n |  |
|  |  | Address Query Section |  |  |  |  |  |
| RLDQ10 | 15 | AFT | O |  | 1 | a/n | A = Rural Route or Route and Box number B = Unnumbered C = Provider assigned house number D = Descriptive |
| RLDQ11 | 16 | SAPR | O |  | 6 | a/n |  |
| RLDQ12 | 17 | SANO | C | Required if TNADDRCKTIND = A and AHN is blank. This field is required when SAPR or SASF are populated. | 10 | a/n |  |
| RLDQ13 | 19 | SASF | O | Optional for TNADDRCKTIND = A and address is numbered. | 4 | a/n |  |
| RLDQ14 | 20 | SASD | O |  | 2 | a/n | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| RLDQ15 | 21 | SASN | C | Required if TNADDRCKTIND = A. This field is required when SASD, SATH, or SASS are populated. | 60 | a/n |  |
| RLDQ16 | 22 | SATH | O |  | 10 | a/n |  |
| RLDQ17 | 23 | SASS | O |  | 2 | a | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| RLDQ18 | 24 | LD1 | C | Required when LV1 is populated, otherwise prohibited. | 4 | a/n | APT LOT RM SLIP UNIT SUIT |
| RLDQ19 | 25 | LV1 | C | Required when LD1 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDQ20 | 26 | LD2 | C | Required when LV2 is populated, otherwise prohibited. | 4 | a/n | FLR |
| RLDQ21 | 27 | LV2 | C | Required when LD2 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDQ22 | 28 | LD3 | C | Required when LV3 is populated, otherwise prohibited. | 4 | a/n | BLDG WNG PIER |
| RLDQ23 | 29 | LV3 | C | Required when LD3 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDQ24 | 30 | CITY | C | Required if TNADDRCKTIND = A. | 32 | a/n |  |
| RLDQ25 | 31 | STATE | C | Required if TNADDRCKTIND = A. | 2 | a |  |
| RLDQ26 | 32 | ZIP | C | Required if TNADDRCKTIND = A. | 12 | a/n |  |
|  |  | Circuit ID Query Section |  |  |  |  |  |
| RLDQ27 | 45 | LSO |  | Required if TNADDRCKTIND = C | 6 | n |  |
| RLDQ28 | 66 | ECCKT |  | Required if TNADDRCKTIND = C | 36 | a/n | Valid ECCKT format: N<n>.AAAA.NNN<nnn>..AA |
|  |  | Raw Loop Data Response (RLDR) |  |  |  |  |  |
|  |  | Administrative Section |  |  |  |  |  |
| RLDR1 | 2 | TXNUM | R |  | 16 | a/n | Echo from query |
| RLDR2 | 3 | D/TSENT XML: MSG_TIMESTAMP | R |  | 17 | a/n | CCYYMMDDHHMinMin Military Time |
| RLDR3 | 4 | TXTYP | R |  | 1 | a/n | R = Raw Loop Data |
| RLDR4 | 5 | TXACT | R |  | 1 | a/n | A = New Inquiry |
| RLDR5 | 7 | CC | R |  | 4 | a/n |  |
|  |  | Raw Loop Data Section This section is filled in only if RESPONSE = G. |  |  |  |  |  |
| RLDR6 |  | ECCKTNUM* | R | Number of ECCKTS: This field gives the total number of ECCKTs being returned. Index of ECCKT/rows in response. | 2 | n |  |
| RLDR7 | 66 | ECCKT* | R |  | 36 | a/n |  |
| RLDR8 | 54 | WTN (Telephone Number)* | R |  | 15 | a/n | Ex: 3035459731 |
| RLDR9 |  | TPRDI* | C | Third Party, Resale, or broadband Indicator: Indicates the third party voice, resale, or broadband Populated if returned | 20 | a/n | Third Party Voice Resale broadband |
| RLDR10 | 15 | AFT* | N | Not used by CenturyLink. | 1 | a/n |  |
| RLDR11 | 16 | SAPR* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 6 | a/n |  |
| RLDR12 | 17 | SANO* | O | ###### Service Address House Number When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 10 | a/n | Example: 3636 |
| RLDR13 | 19 | SASF* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 4 | a/n |  |
| RLDR14 | 20 | SASD* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 2 | a | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| RLDR15 | 21 | SASN* | O | ###### Service Address Street Name When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 60 | a/n | Example: Roundtree Ct |
| RLDR16 | 22 | SATH* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 10 | a/n |  |
| RLDR17 | 23 | SASS* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 2 | a | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| RLDR18 | 24 | LD1* | C | Required when LV1 is populated, otherwise prohibited. | 4 | a/n | APT LOT RM SLIP UNIT SUIT |
| RLDR19 | 25 | LV1* | C | Required when LD1 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDR20 | 26 | LD2* | C | Required when LV2 is populated, otherwise prohibited. | 4 | a/n | FLR |
| RLDR21 | 27 | LV2* | C | Required when LD2 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDR22 | 28 | LD3* | C | Required when LV3 is populated, otherwise prohibited. | 4 | a/n | BLDG WNG PIER |
| RLDR23 | 29 | LV3* | C | Required when LD3 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDR24 | 30 | CITY* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 32 | a/n | Example: Greenwood Village |
| RLDR25 | 31 | STATE* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 2 | a | Example: CO |
| RLDR26 | 32 | ZIP* | O | If ZIP is present in address query, echo from address query; otherwise populate as null. | 12 | a/n | Example: 80202-1234 |
| RLDR27 | 60 | LST* | O | Local Serive Termination: This is the wire center CLLI Code. | 8 | a/n | Example: BLDRCOMA |
| RLDR28 |  | MLTDIST * | O | MLT Distance: The mechanized loop test distance. | 10 | n | Example: 34766 |
| RLDR29 | 90 | AFS* | O | ###### Assembled Facilities Status | 4 | a/n | A = Working B = Pending-in C = Pending-out D = Connect through E = Connected facility F = Spare G = Partially connected facility R = Remote Working |
| RLDR30 |  | SEGMENTNUM ** | C | Segment Number: The actual segment number Populated if returned. | 2 | n | 1-99 |
| RLDR31 |  | TERMINAL_ID** | O | Terminal ID: The terminal identification. | 50 | a/n |  |
| RLDR32 |  | BRIDGE_TAP_OFFSET_DESC** | O | Bridged Tap Offset: This provides information about bridge taps. | 150 | a/n |  |
| RLDR33 |  | MAKE_UP_DESC** | O | Make Up Description: This provides data about loop makeup. | 400 | a/n |  |
| RLDR34 | 91 | CA** | O | Cable Name: The cable identifier being queried. If CA** is FTH you cannot resell data services. | 10 | a/n | Valid Cable Name Prefixes BB = Broadband Network Unit VDSL Fiber to the Curb CH = Carrier Hub CMP = Campus Wire Environment DSL = Digital Subscriber Line, ADSL DSLB = Next Level Digital Subscriber Line, ADSL NLC Broadband Network Unit ADSL Fiber to the Curb EX = Remote Switch Unit / Optical Remote Module FCP = Field Connection Point FH = Fiber Hub FTH = Fiber to the Premises IPG = Integrated Digital Loop Carrier IPGXB = Cross wire center boundary IDLC L = Fiber LH = Fiber Hub LPGB = VDSL-equipped USAM (universal service access multiplexer) VDSL-equipped BSAM (broadband services access multiplexer) MTE = Multi-tenant Environment PG = Pair Gain PGBR#a = BR1/10 ISDN single-system channel bank or side A of a dual- system channel bank PGBR#b = Side B of a dual system channel bank PGL = Next Level DLC Applications PWR = Power Cable RMU = Remote Measurement Unit T = Interoffice Cable UDC = Universal Digital Carrier UDF = Unbundled Dark Fiber |
| RLDR35 | 101 | TRMED** | O | Transmission Medium Type: This identifies the type of pair gain, if present. | 9 | a/n |  |
| RLDR36 | 95 | CHAN/PAIR** | O | Pair Number: The unique number of pair being queried. | 5 | a/n |  |
| RLDR37 | 84 | LCQ ** | O | ###### Load Coil Quantity | 1 | n | Example: 2 |
| RLDR38 | 85 | LCT** | O | ###### Load Coil Type | 5 | a/n |  |
|  |  | Response Section |  |  |  |  |  |
| RLDR39 | 139 | RESPC* | C | Error Code: Indicates a predetermined error code. | 3 | n |  |
| RLDR40 | 140 | RESPD* | C | Error Message: Indicates additional information about the error. | 80 | a/n |  |
| RLDR41 | 141 | PRESPC* | O |  | 14 | a/n | Format (Character positions): 1-3 : Form Name 4-11 : Field Name 12-14 : Error Code |
| RLDR42 | 142 | PRESPD* | O | Response G = Good B = Bad M = Mixed MIXTYP P = Partial response Note: Partial response must identify the WTN It is associated with | 264 | a/n |  |